Packet Tracer - Configuring DHCP Using Cisco IOS

Topology

Addressing Table

| Device | Interface | IPv4 Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- |
| R1 | G0/0 | 192.168.10.1 | 255.255.255.0 | N/A |
|  | S0/0/0 | 10.1.1.1 | 255.255.255.252 | N/A |
| R2 | G0/0 | 192.168.20.1 | 255.255.255.0 | N/A |
|  | G0/1 | DHCP Assigned | DHCP Assigned | N/A |
|  | S0/0/0 | 10.1.1.2 | 255.255.255.252 | N/A |
|  | S0/0/1 | 10.2.2.2 | 255.255.255.252 | N/A |
| R3 | G0/0 | 192.168.30.1 | 255.255.255.0 | N/A |
|  | S0/0/1 | 10.2.2.1 | 255.255.255.0 | N/A |
| PC1 | NIC | DHCP Assigned | DHCP Assigned | DHCP Assigned |
| PC2 | NIC | DHCP Assigned | DHCP Assigned | DHCP Assigned |
| DNS Server | NIC | 192.168.20.254 | 255.255.255.0 | 192.168.20.1 |

Objectives

Part 1: Configure a Router as a DHCP Server

Part 2: Configure DHCP Relay

Part 3: Configure a Router as a DHCP Client

Part 4: Verify DHCP and Connectivity

© 2013 Cisco and/or its affiliates. All rights reserved. This document is Cisco Public. Page 1 of 3

Packet Tracer - Configuring DHCP Using Cisco IOS

Scenario

A dedicated DHCP server is scalable and relatively easy to manage, but can be costly to have one at every  location in a network. However, a Cisco router can be configured to provide DHCP services without the need  for a dedicated server. As the network technician for your company, you are tasked with configuring a Cisco  router as a DHCP server to provide dynamic allocation of addresses to clients on the network. You are also  required to configure the edge router as a DHCP client so that it receives an IP address from the ISP network.

Part 1: Configure a Router as a DHCP Server

Step 1: Configure the excluded IPv4 addresses.

Configure R2 to exclude the first 10 addresses from the R1 and R3 LANs. All other addresses should be  available in the DHCP address pool.

Step 2: Create a DHCP pool on R2 for the R1 LAN.

a. Create a DHCP pool named R1-LAN (case-sensitive).

b. Configure the DHCP pool to include the network address, the default gateway, and the IP address of the  DNS server.

Step 3: Create a DHCP pool on R2 for the R3 LAN.

a. Create a DHCP pool named R3-LAN (case-sensitive).

b. Configure the DHCP pool to include the network address, the default gateway, and the IP address of the  DNS server.

Part 2: Configure DHCP Relay

Step 1: Configure R1 and R3 as a DHCP relay agent.

Step 2: Set PC1 and PC2 to receive IP addressing information from DHCP. Part 3: Configure R2 as a DHCP Client

a. Configure the Gigabit Ethernet 0/1 interface on R2 to receive IP addressing from DHCP and activate the  interface.

Note: Use Packet Tracer’s Fast Forward Time feature to speed up the process or wait until R2 forms an  EIGRP adjacency with the ISP router.

b. Use the show ip interface brief command to verify that R2 received an IP address from DHCP. Part 4: Verify DHCP and Connectivity

Step 1: Verify DHCP bindings.

R2# show ip dhcp binding

IP address Client-ID/ Lease expiration Type

Hardware address

192.168.10.11 0002.4AA5.1470 -- Automatic 192.168.30.11 0004.9A97.2535 -- Automatic

© 2013 Cisco and/or its affiliates. All rights reserved. This document is Cisco Public. Page 2 of 3

Packet Tracer - Configuring DHCP Using Cisco IOS

Step 2: Verify configurations.

Verify that PC1 and PC2 can now ping each other and all other devices.

© 2013 Cisco and/or its affiliates. All rights reserved. This document is Cisco Public. Page 3 of 3